# HT12055 Self-Powered Module

This self-powered interface is packaged in a small in-line module complete with industry standard cash drawer connectors:-

HT12055  – RS232 to RJ11 cash drawer connector.

Features:

- Power derived from PC serial connector  i.e. COM1

- Opens 12 or 24 volt cash drawers

- Uses industry standard ‘EPSON’ compatible RJ11 or ‘Cash Bases’ dual-voltage DB9F cash drawer connector.

- Can open drawer repetitively every 5 seconds maximum, after initial charge up time of 20 seconds maximum from computer switch on

- Returns cash drawer open/closed status via CTS signal

- PC cable integral  (9 way D type)

- Can be set to open cash drawer on ONE, TWO or more characters (set by baud rate) **

- Ignores power on/off spikes from PCs

- Housed in a small ABS module 110 x 57 x 22 mm

- No switches / links to set-up, just plug in and use

- Quieter solenoid operation via power conservation which removes power when drawer is seen to have opened

Set Up:

Intended for use with a spare COM port that can be dedicated to open a Cash Drawer and any software package that can ‘print’ to that COM port.

The actual characters printed do not matter, however, the number of characters needed depends on the number of zeros in the binary representation of the character – ideal character is the NUL character (binary 00000000). For example a single NUL character at 300 baud will open the cash drawer, two NUL characters at 600 baud will open the cash drawer etc. This will vary slightly, depending on the combination of P.C. and cash drawer being used.

A simple set-up can be achieved by sending the desired characters and increasing the baud rate step by step until the cash drawer no longer opens, then decrease the baud rate by two steps to allow a margin of error.

** Typical settings: (depending on actual characters) are:

For single characters use 300 baud or slower

For two characters use 600 baud or slower

For five characters use 2400 baud or slower

00  00 		opens at 1200 baud

07 		opens at 300 baud

07 07 		opens at 600 baud

1B 30 		opens at 600 baud

1B 70 00 55 00 	opens at 2400 baud

Alternatively send approximately twenty NUL characters at 9600 baud

DTR must remain set High (This should be set automatically when addressing the COM port).

CTS may be used to read the drawer open / closed condition.

ECD Ltd.								VFD 07-06-2000